|  | AUSGABE (des Beilageblattes) 2026-07-29 In diesem Decoder ist folgendes ZIMO Sound-Projekt geladen: Regelspurddampflokomotive CabForward AC5 – Version 30.23 |
| --- | --- |

Southern Pacific Railroad's AC-5 class of steam locomotives was the railroad's second class of 4-8-8-2 cab forward locomotives. They were built between July and September 1929 and placed in service soon after construction. The first locomotive of this class, number 4110, holds the dubious honor of being the first 4-8-8-2 cab forward locomotive to be scrapped when it befell this fate on February 3, 1953, at the railroad's Sacra-mento, California, shops. The rest of the class were removed from service and scrapped by mid-1955. None of this class was preserved.

Der Einbau der ZIMO Sound Decoder MX64x (H0) und MX69x (Großbahn) mit diesem Projekt kann in alle passenden Modelle erfolgen.

ACHTUNG: Nach dem Einbau des Decoders ist eine Messfahrt notwendig  –>  CV  # 302 = 75

In diesem Projekt sind folgende Funktionszuordnungen getroffen:

F-Taste                     Einrichtung                                               am Funktionsausgang            Sound-Funktion

| F0 | Licht | FA0 (gelb/weis) | Lichtmaschine |
| --- | --- | --- | --- |
| F1 |  | FA1 |  |
| F2 |  |  |  |
| F3 |  |  | Pfeife 1 |
| F4 | ABV/ Rangiergang |  |  |
| F5 |  |  | Kohleschaufeln |
| F6 |  |  | Entwässern |
| F7 |  |  | Glocke |
| F8 |  |  | SOUND ON/OFF |
| F9 |  |  | Pfeife 2 |
| F10 |  |  | Pfeife 3 |
| F11 |  |  | Bläser |
| F12 |  |  | Wasserfassen |
| F13 |  |  | Entlüften |
| F14 |  |  | Injektor |
| F15 |  |  | Luftpumpe |
| F16 |  |  | Überdruck |
| F17 |  |  | Kupplung 1 |
| F18 |  |  | Kupplung 2 |
| F19 |  |  |  |

Für Mehrfachtraktion in CV154 Bit 1 setzen!